VnmrJ / VNMR FID Data Format

With very minor exceptions (which are irrelevant in the context of data

importing) we have not changed the binary data format since the introduction

of VNMR almost 20 years ago, and this data format is described exhaustively in

section 5 "Parameters and Data" of the User Programming Manual. For FIDs, the

key is that FID files have a 32-byte data header with the following structure:

struct fileHeader

{

long nblocks, ntraces, np, ebytes, tbytes, bbytes;

short vers_id, status;

long nblockheaders;

};

where the elements are defined as follows:

* "nblocks" is the number of data blocks in the file,

* "ntraces" is the number of (FID) "traces" per block (high resolution

* spectra with nf=1 have one FID / "trace" per block, multi-echo FIDs such

* as imaging FIDs acquired in "compressed" mode may have multiple FIDs /

* "traces" per block),

* "np" is the number of (real, not complex) points per "trace" (typically

* equal to the np parameter),

* "ebytes" is the number of bytes per "element" (2 for 16-bit data, 4 for

* 32-bit data),

* "tbytes" is the number of bytes per "trace", i.e.,

fheader.ebytes * fheader.np

* "bbytes" is the number of bytes per block, i.e., in "C speak":

fheader.ebytes * fheader.np * fheader.ntraces +

sizeof(struct blockHeader)

(struct blockHeader is discussed below).

* "vers_id" may be used for the VnmrJ version ID (you may find this set to

* zero - it's not really used in our software);

* "status" indicates the status of the data; the value of "status" is a

* flag field similar to the protection bits in the VNMR parameter

* definition:

o bit 0 (0x1) indicates the presence of data

o bit 1 (0x2) would indicate "spectrum" (as opposed to "FID")

o bit 2 (0x4) indicates 32-bit (as opposed to 16-bit) data

o bit 3 (0x8) indicates (32-bit, IEEE format) floating point data

(in which case the value of bit 2 is irrelevant)

o bit 4 (0x10) indicates complex (as opposed to real) data

o bit 5 (0x20) indicates hypercomplex data (transformed nD data

only)

o bit 6 (0x40) is unused up to VnmrJ 1.1D

for the other bit fields see the User Programming Manual. In a typical

VnmrJ 1.1D FID you may find the "status" field to have a value of 0x15, i.e.,

0x1 (data present) + 0x4 (32-bit, i.e., dp='y') + 0x10 (complex)

* "nblockheaders" is the number of block headers (always 1 for FIDs)

The block header that follows has a size of 28 bytes and is defined as

struct blockHeader

{

short scale, status, index, mode;

long ctcount;

float lpval, rpval, lvl, tlt;

};

Where the elements have the following meaning:

* "scale" is the number of downscaling operations (non-circular, bit-wise

* right-shifts, or divisions by 2) while accumulating data (particularly

* when acquiring with a very large nt and dp='n') in the given block (see

* the article "How Does the Autogain Work?" in Varian NMR News

* 1999-09-11);

* "status" in FIDs is usually the same as in the file header, see above;

* "ctcount" is the value of ct for the associated FID block;

* "lvl" and "tlt" are used to store the d.c. offsets (as determined via

* the noise sampling prior to acquisition) in channels A and B in the case

* of nt=1, see Varian NMR News 2001-02-02.

For explanation on the other fields see the User Programming Manual.

The first block header is followed by the first data block, and each of the

subsequent data blocks is preceded by its own block header (or the number of

block headers in special cases of processed nD data, as indicated in the file

header). In all cases - and most certainly in the case of FIDs - the structure

of the binary data (FID) file, i.e.,

* the number of blocks,

* the number of traces (FIDs / echoes) per block,

* the number of "elements" (numbers) per trace (FID), and

* the format of an element (32-bit IEEE floating point vs. 32-bit integer

* vs. 16-bit integer)

is completely and accurately described in the data headers. The byte order is

the one used with the Motorola MC68xxx, the Motorola PowerPC (PPC) and Sun's

SPARC family of processors, even if the data were acquired using VnmrJ_LX on a

system with an Intel / Linux based PC as spectrometer host, see Varian NMR

News 2005-01-22.